- Task 4.1 Calibration

- Overview of the calibrating facility

FACILITY ID: ACQUA ALTA research tower

BRIEF DESCRIPTION: The "Acqua Alta" research tower is installed 15 km offshore Venice, Italy, in 16 m of water (MLLW). Autonomous instrumentations cover atmospheric and hydrological parameters with a series of meteorological stations (wind, air and water temperature, atmospheric pressure, humidity, rain) and a series of oceanographic instruments (waves, currents along the column with ADCP, temperature at surface and bottom, salinity, turbidity, oxygen, chlorophyll a and sea level).

- Contact Details

NAME/DESIGNATION (if any): ////////////

- MANAGING INSTITUTE/ORGANIZATION:  CNR National Research Council of Italy

DEPARTMENT (if any):  ISMAR Institute of Marine Sciences (headquarter)

- ADDRESS: Castello 1364/a, Venice

- COUNTRY:  ITALY

- TEL:  +390412404711

- FAX:  +390415204126

Name of contact-person:  Mauro Bastianini

- E-mail: mauro.bastianini@ismar.cnr.it

- Part a: General Information

1. Does your calibrating facility possess a well-defined organizational framework with

Dedicated staff?										No

Clear hierarchy?										No

Transparent chain of responsibility for management, technical/scientific

and operational decisions)?								No

(If No to any of the above, please provide a brief description of how your facility is organized below)

Local labs are dedicated to calibration processes no certification nor dedicated staff.

Moreover, sensors are sent regularly to the manufacturer for calibration.

2. Briefly describe the size and nature of the annual operating budget of your facility.

Is it funded by your Institute/Centre?							Yes

If Yes, is the funding constant?							            Yes

Is it funded by Projects?									Yes

Is there separate funding for upgrading or acquiring new instrumentation, etc.?	No

(Kindly provide an estimate of the annual operating budget and any additional information you think may be helpful below) ___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

3. Does your facility employ Quality Management Standards - ISO 9000:2000,

ISO 10012, Good Laboratory Practice (GLP), and the like - to its calibration

systems? 								                                 No

(If Yes, please specify below)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

4. Does your facility possess any kind of accreditation for the calibrations?		No

(If Yes, please specify the parameter/s or measurand/s concerned, the kind

of accreditation and the issuing body below)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

5. Does your facility actively endorse a policy of continual training/education of

personnel actively involved in calibration activity? 					No

(If Yes, please provide a brief description of the kind of activities promoted below)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

6. Does your facility maintain a documented in-house Quality Assurance Programme?	No

7. Does your facility maintain a formal Quality Manual (containing, at the

very least, listings and descriptions of equipment and procedures,

maintenance/calibration records and certificates for instrumentation, and

safety precautions and regulations)?                                                                              No

8. Does your facility make use of control charts (Shewhart Charts, other) for

Quality Control purposes?								No

(If Yes, please give details below) ___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

9. Can your facility assure an effective traceability chain to primary standards or,

in their absence, to conventionally accepted reference material (certified or

otherwise)?										No

10. Does your facility furnish uncertainty estimations for its calibration systems?		No

11. Does your facility maintain links of any kind with the National Metrology

Institute/s (NMI/s) of your country?							Yes

(If Yes, please describe the nature of the relationship/s below)

Models used for wave forecast

12. In the list of sensors below, please indicate only the ones that you currently never calibrate yourselves; in each case, kindly report the calibration provider (manufacturer, other) and the typical calibration interval (trimonthly, half-yearly, yearly, other) you are presently employing.

- Physical sensors for:

× Temperature, monthly ourselves, Italian distributor yearly

× Conductivity monthly ourselves (Salinity), Italian distributor yearly

  Dissolved oxygen

 Water Currents, __________________

×  Pressure, monthly ourselves, Italian distributor yearly

- Optical sensors for:

×  Dissolved oxygen, monthly ourselves, Italian distributor yearly        (added in this list)

 Chlorophyll a, __________________

×  Turbidity, monthly ourselves, Italian distributor yearly

 Photosynthetically Active Radiation (PAR), __________________

- Chemical sensors for:

 Phosphates, __________________       Silicates, __________________

 Nitrates, __________________            Nitrites, __________________

 Ammonia, __________________         Dissolved oxygen, __________________

pH, __________________        Total alkalinity, __________________

 Total carbon dioxide, __________________

Dissolved organic carbon, __________________

Total organic carbon, __________________

- Task 4.1.1 Physical Sensors

(* Please provide a separate sheet for each parameter)

Part b: Calibration

Parameter/measurand*: Temperature

Unit of measurement: Celsius degree

Range: -5 +50

Accuracy: ±0.10

Precision: 0.01

Calibration uncertainty (if available): ______________________________

1. How often do you calibrate the sensor/s or sensor system/s you are presently using for the specified parameter/measurand: please list the typical calibration interval/s you are employing; note that if you are calibrating irregularly, kindly specify why and when (e.g. before a deployment, following a malfunction, etc.).

following a malfunction

2. Please provide a brief description of the calibration setup, including a list of the principal equipment, reference material (certified and/or conventionally accepted) and instrumentation involved in a typical calibration operation.

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

3. Do you employ reference material which are mutable or unstable

(e.g. secondary standards, reagent solutions, gas mixtures,

pressure generators, etc.) to calibrate the sensor/s or sensor system/s

you are presently using for the specified parameter/measurand.			No

4. In your view, does your facility ensure an effective traceability chain for the

specified parameter/measurand?								No

5. Please provide a brief description of the procedures employed to ensure adherence of the performances of the principal equipment and reference instrumentation of the calibration setup to factory specifications (in-house monitoring of performance, in loco re-calibration, servicing by the manufacturer, etc.).

in-house monitoring of performance

6. Does your facility maintain a Manual with a description of the calibration method

and the measuring procedures, together with details of sample treatment and

preparation when these steps are present?						 No

7. In your view, is regular factory calibration/servicing necessary to obtain

optimal performances from your sensors/instrumentation for the

specified parameter/measurand in the field?						No

8. Do you perform field calibrations for the specified parameter/measurand?		Yes

(If Yes, please provide a brief description of the method and procedures)

Cross check with other sensors

9. Does your facility perform:

- internal quality audits to monitor and assess its

calibration system for the specified parameter?						No

-    independent quality audits to monitor and assess its

calibration system for the specified parameter?						No

10. Does your facility actively maintain an archive containing issued calibration

reports/certificates for the specified parameter/measurand?				 No

11. Do you have any suggestions or ideas for improving the quality of your

calibrations for any particular sensor/sensor system you are presently using

for the specified parameter/measurand (e.g. innovative reference material,

modifications to existing methodologies or new methodologies

you have developed, etc.)?								No

12. Do you have any suggestions or ideas for improving the general quality

of the calibration of sensors or instruments for measuring the specified

parameter/measurand (e.g. testing and promoting the use of new

reference material, development of new methodologies, etc.)?				 No

- Task 4.1.1 Physical Sensors

(* Please provide a separate sheet for each parameter)

Part b: Calibration

Parameter/measurand*: Conductance

Unit of measurement: PSU

Range: 0-100 mS/cm

Accuracy: ±0.20

Precision: 0.01

Calibration uncertainty (if available): ______________________________

1. How often do you calibrate the sensor/s or sensor system/s you are presently using for the specified parameter/measurand: please list the typical calibration interval/s you are employing; note that if you are calibrating irregularly, kindly specify why and when (e.g. before a deployment, following a malfunction, etc.).

every 15-20 days

2. Please provide a brief description of the calibration setup, including a list of the principal equipment, reference material (certified and/or conventionally accepted) and instrumentation involved in a typical calibration operation.

Water samples analysis using standard sea water

3. Do you employ reference material which are mutable or unstable

(e.g. secondary standards, reagent solutions, gas mixtures,

pressure generators, etc.) to calibrate the sensor/s or sensor system/s

you are presently using for the specified parameter/measurand.			No

4. In your view, does your facility ensure an effective traceability chain for the

specified parameter/measurand? 								No

5. Please provide a brief description of the procedures employed to ensure adherence of the performances of the principal equipment and reference instrumentation of the calibration setup to factory specifications (in-house monitoring of performance, in loco re-calibration, servicing by the manufacturer, etc.).

in-house monitoring of performance

6. Does your facility maintain a Manual with a description of the calibration method

and themeasuring procedures, together with details of sample treatment and

preparation when these steps are present?						No

7. In your view, is regular factory calibration/servicing necessary to obtain

optimal performances from your sensors/instrumentation for the

specified parameter/measurand in the field? 						No

8. Do you perform field calibrations for the specified parameter/measurand? 		Yes

(If Yes, please provide a brief description of the method and procedures)

Cross check with other sensors

9. Does your facility perform:

- internal quality audits to monitor and assess its

calibration system for the specified parameter?						No

-    independent quality audits to monitor and assess its

calibration system for the specified parameter?						No

10. Does your facility actively maintain an archive containing issued calibration

reports/certificates for the specified parameter/measurand? 				No

11. Do you have any suggestions or ideas for improving the quality of your

calibrations for any particular sensor/sensor system you are presently using

for the specified parameter/measurand (e.g. innovative reference material,

modifications to existing methodologies or new methodologies

you have developed, etc.)?								No

12. Do you have any suggestions or ideas for improving the general quality

of the calibration of sensors or instruments for measuring the specified

parameter/measurand (e.g. testing and promoting the use of new

reference material, development of new methodologies, etc.)?				No

- Task 4.1.1 Physical Sensors

(* Please provide a separate sheet for each parameter)

Part b: Calibration

Parameter/measurand*: Depth

Unit of measurement: m

Range: 0-100 m

Accuracy: ±0.10

Precision: 0.1

Calibration uncertainty (if available): ______________________________

1. How often do you calibrate the sensor/s or sensor system/s you are presently using for the specified parameter/measurand: please list the typical calibration interval/s you are employing; note that if you are calibrating irregularly, kindly specify why and when (e.g. before a deployment, following a malfunction, etc.).

following a malfunction

2. Please provide a brief description of the calibration setup, including a list of the principal equipment, reference material (certified and/or conventionally accepted) and instrumentation involved in a typical calibration operation.

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

3. Do you employ reference material which are mutable or unstable

(e.g. secondary standards, reagent solutions, gas mixtures,

pressure generators, etc.) to calibrate the sensor/s or sensor system/s

you are presently using for the specified parameter/measurand.                                      No

4. In your view, does your facility ensure an effective traceability chain for the

specified parameter/measurand? 								   No

5. Please provide a brief description of the procedures employed to ensure adherence of the performances of the principal equipment and reference instrumentation of the calibration setup to factory specifications (in-house monitoring of performance, in loco re-calibration, servicing by the manufacturer, etc.).

in-house monitoring of performance

6. Does your facility maintain a Manual with a description of the calibration method

and themeasuring procedures, together with details of sample treatment and

preparation when these steps are present?						No

7. In your view, is regular factory calibration/servicing necessary to obtain

optimal performances from your sensors/instrumentation for the

specified parameter/measurand in the field? 						No

8. Do you perform field calibrations for the specified parameter/measurand? 		Yes

(If Yes, please provide a brief description of the method and procedures)

Cross check with other sensors

9. Does your facility perform:

- internal quality audits to monitor and assess its

calibration system for the specified parameter?						No

-    independent quality audits to monitor and assess its

calibration system for the specified parameter?						No

10. Does your facility actively maintain an archive containing issued calibration

reports/certificates for the specified parameter/measurand? 				No

11. Do you have any suggestions or ideas for improving the quality of your

calibrations for any particular sensor/sensor system you are presently using

for the specified parameter/measurand (e.g. innovative reference material,

modifications to existing methodologies or new methodologies

you have developed, etc.)?								No

12. Do you have any suggestions or ideas for improving the general quality

of the calibration of sensors or instruments for measuring the specified

parameter/measurand (e.g. testing and promoting the use of new

reference material, development of new methodologies, etc.)?				No

- Task 4.1.2 Optical Sensors

(* Please provide a separate sheet for each parameter)

Part b: Calibration

Parameter/measurand*: Dissolved oxygen

Unit of measurement: mg/L

Range: 0-30

Accuracy: 0.01

Precision: 0.1

Calibration uncertainty (if available): ______________________________

1. How often do you calibrate the sensor/s or sensor system/s you are presently using for the specified parameter/measurand: please list the typical calibration interval/s you are employing; note that if you are calibrating irregularly, kindly specify why and when (e.g. before a deployment, following a malfunction, etc.).

15-20 days

2. Please provide a brief description of your calibration setup for the specified parameter/measurand, including a list of the principal equipment, reference material (certified and/or conventionally accepted) and instrumentation involved in a typical calibration operation.

calibration in 100% sat air

3. Do you employ reference material which are mutable or unstable

(e.g. biological cultures, optically-sensitive pigment extracts, etc.)

tocalibrate the sensor/s or sensor system/s you are presently using for

the specified parameter/measurand.							Yes

(if Yes, please list the types of this kind of reference material you are employing; kindly specify also the measures you take to guarantee the reliability of the reference material in terms of batch-to-batch uniformity of characteristics)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

4. In your view, does your facility ensure an effective traceability chain for the

specified parameter/measurand? 								No

5. Please provide a brief description of the procedures employed to ensure adherence of the performances of the principal equipment and reference instrumentation of the calibration setup to factory specifications (in-house monitoring of performance, in loco re-calibration, servicing by the manufacturer, etc.).

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

6. Does your facility maintain a Manual with a description of the calibration method

and themeasuring procedures, together with details of sample treatment and

preparation when these steps are present?						No

7. In your view, is regular factory calibration/servicing necessary to obtain

optimal performances from your sensors/instrumentation for the

specified parameter/measurand in the field? 						No

(If Yes, please provide details of the sensors/instrumentation, indicating also the intervals you recommend for factory calibration/servicing, below)

8. Do you perform field calibrations for the specified parameter/measurand? 		No

(If Yes, please provide a brief description of the method and procedures)

9. Does your facility perform:

- internal quality audits to monitor and assess its

calibration system for the specified parameter?					            No

-    independent quality audits to monitor and assess its

calibration system for the specified parameter?					            No

10. Does your facility actively maintain an archive containing issued calibration

reports/certificates for the specified parameter/measurand? 			            No

11. Do you have any suggestions or ideas for improving the quality of your

calibrations for any particular sensor/sensor system you are presently using

for the specified parameter/measurand (e.g. innovative reference material,

modifications to existing methodologies or new methodologies

you have developed, etc.)? 								No

12. Do you have any suggestions or ideas for improving the general quality

ofthe calibrationof sensors or instruments for measuring the specified

parameter/measurand (e.g. testing and promoting the use of new

reference material, development of new methodologies, etc.)? 				No

- Task 4.1.2 Optical Sensors

(* Please provide a separate sheet for each parameter)

Part b: Calibration

Parameter/measurand*: Turbidity

Unit of measurement: NTU

Range: 0-1000

Accuracy: 5%

Precision: 0.1

Calibration uncertainty (if available): ______________________________

1. How often do you calibrate the sensor/s or sensor system/s you are presently using for the specified parameter/measurand: please list the typical calibration interval/s you are employing; note that if you are calibrating irregularly, kindly specify why and when (e.g. before a deployment, following a malfunction, etc.).

15-20 days

2. Please provide a brief description of your calibration setup for the specified parameter/ measurand, including a list of the principal equipment, reference material (certified and/or conventionally accepted) and instrumentation involved in a typical calibration operation.

calibration in known formazide solution

3. Do you employ reference material which are mutable or unstable

(e.g. biological cultures, optically-sensitive pigment extracts, etc.)

to calibrate the sensor/s or sensor system/s you are presently using for

the specified parameter/measurand.                                                                                   Yes

(ifYes, please list the types of this kind of reference material you are employing; kindly specify also the measures you take to guarantee the reliability of the reference material in terms of batch-to-batch uniformity of characteristics)

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

4. In your view, does your facility ensure an effective traceability chain for the

specified parameter/measurand? 								No

5. Please provide a brief description of the procedures employed to ensure adherence of the performances of the principal equipment and reference instrumentation of the calibration setup to factory specifications (in-house monitoring of performance, in loco re-calibration, servicing by the manufacturer, etc.).

___________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(Add lines as necessary)

6. Does your facility maintain a Manual with a description of the calibration method

and themeasuring procedures, together with details of sample treatment and

preparation when these steps are present?						No

7. In your view, is regular factory calibration/servicing necessary to obtain

optimal performances from your sensors/instrumentation for the

specified parameter/measurand in the field? 						No

8. Do you perform field calibrations for the specified parameter/measurand? 		No

9. Does your facility perform:

- internal quality audits to monitor and assess its

calibration system for the specified parameter?						No

-    independent quality audits to monitor and assess its

calibration system for the specified parameter?						No

10. Does your facility actively maintain an archive containing issued calibration

reports/certificates for the specified parameter/measurand? 				No

11. Do you have any suggestions or ideas for improving the quality of your

calibrations for any particular sensor/sensor system you are presently using

for the specified parameter/measurand (e.g. innovative reference material,

modifications to existing methodologies or new methodologies

you have developed, etc.)? 								No

12. Do you have any suggestions or ideas for improving the general quality

of the calibration of sensors or instruments for measuring the specified

parameter/measurand (e.g. testing and promoting the use of new

reference material, development of new methodologies, etc.)? 				No

- Submitted on 25/11/11

Compiled by: Mauro Bastianini